# 2026 Shell Eco Marathon Debrief Document

# Planned improvements to the car

## Mechanical changes to car (by priority):

- Modify steering and front brakes to reduce knocking

  - Make sure the steering column is locked with C clips and cannot be accidentally disassembled

- Remake suspension plates to remove camber from wheels

  - Make sure the wheels are correctly pressurized

  - Camber adjustability would be good

- Move windshield hinge to front of windshield (makes entering/dismounting easier)

  - Or a 4-bar linkage

- Improve brake ergonomics

  - Improve cable routing under the upper floor and also out the sides so that it doesn’t pull on floor and side panels

  - Bleed rear brakes

  - Actual brake pedal

- Remake front brake caliper mounts to fit

  - Some level of adjustability would be good

- Remove front and/or rear suspension to reduce weight (if rules permit)

  - Redesign suspension with shorter, softer springs

- Standardize one or two screw sizes so we don’t have to spend forever looking for the right bits

- Improve driver comfort (how?)

  - Mount steering, pedals, harness, buttons, seat, etc. around the driver

  - Better seat (hammock seat was pretty cool)

- Removable side panels would be nice to make serviceability easier

- Fix the floor

- If necessary, improve airflow in fuel cell area with more strategic scoops/vent holes

## Changes to electrical system (by priority):

- Identify and fix cause of power shutoffby

- Replace existing motor with more powerful motor

- Better cable routing

## Changes to fuel cell system (by priority):

- Make the back panel that all the plumbing is mounted to easier to remove

- Figure out the correct tank dimensions

- Pay attention to conical vs cylindrical threading

# Logistical changes for next year

## Updated packing list

- 3D printer and filament!

## Stuff to buy

New helmet

New regulator

L connectors for fuel cell

Lower level gas can

More drills

Complete sets of tools

More sharpie colors

More wire

More swagelock tube bending tools

PTFE tubing

The right brake fluid

Rolling chassis table

Tool chest/pack out

New motor

C clips

Chairs

## General Advice

Label stuff

Learn how to mount stuff on tubes

Learn how to tolerance 3D prints

Get sponsors

Read the whole rule book

Get more camping chairs