# Aluminium Connectors

### Lathe

  - Insert 1 in stock

  - Zero out lathe

    - LENGTH AND DIAMETER

  - Turn the smaller wheel to 240˚, and lathe to approx 2 in (about 20 revolutions on dial)

  - Test fit

    - It will probably not fit if you did it correctly

  - Turn wheel approx 5˚ and lathe again, test fit

  - If it fits great, if it doesn’t turn wheel a little bit and repeat

  - Go to end of material, approx 0.3 in (does not need to be precise), turn on lathe, then carve a small groove out by only turning the small wheel

    - Only do so a couple degrees

  - Go about 0.2 in above groove and cut another one

  - Get metal rasp and remove burrs on the lather

    - Trust it's a lot easier now than after

  - Hollow out the middle using the drill on the lather

    - Use the bit that’s approx 0.39 in

    - Use the cinnamon gunk (not actual cinnamon just smells like it) here it makes the lather a lot happier

  - Carefully cut the lathed portion off leaving the corresponding amount of stock for application

    - 1.300 in for vertical members

    - 2.300 in for one ended horizontal members

    - 4.400 in for double ended horizontal members

  - For double ended members, repeat process on other side

  - WHEN REMOVING BE CAREFUL METAL IS HOT

  - The connector should look something like this:

OR

### Milling

  - WEAR LONG SLEEVES, HOOD, AND COVER COLLARBONE

    - Just trust it's not pleasant when hot and sharp metal pieces are flying at your face

  - Set up connector in square holder

    - Will need the 0.75 size round holder that goes into the square block

    - Lathed connector will go in end opposite to the thing you screw on the bottom of the block

    - Will need to tighten until you can't tighten anymore with the wrench

  - If necessary, clean machine so no shavings are where the block will go

    - Use the tiny brush try to prevent flying metal pieces

  - Place block holding connector in the machine

    - make sure block is flush to the side of the clamps using the small metal square

    - Run your finger across the seam to verify that it is flush

    - Tighten

  - If necessary, put the right bit into the machine

    - Ask J to show you how

    - Need to hold the machine’s brake and turn the bolt on top using the wrench

      - I can’t reach so I usually either find Zucker’s stool or ask J

  - If necessary, raise the bed of the machine by turning the giant chrome removable crank until the connector is just below the drill bit

    - It's the left crank on the bottom of the bed

    - Give yourself a couple inches of bail room

  - If necessary, change the speed of the machine using the dial on the front

    - Ask J which speed to use

    - Want the machine on high

    - Want something around 1200

  - Zero the machine (Z-axis)

    - Ask J to show you how

    - Hit the DRO button to get to the right menu

    - Crack various wheels until connector is under the drill bit

    - Put tiny piece of paper on the top of the connector

    - Crank the giant lever on the right side

      - It looks like this:

      - (refrain from saying “Wrong lever Kronk”)

    - Crank lever until you can still pull the piece of paper out but there is a little bit of resistance

    - Turn the smaller black lever so that it locks

      - It looks like this:

    - On the screen hit “Z” and then “abs zero”

  - Use the wheels to move the connector from under the drill

  - Rotate the giant lever out and crank until the screen says -0.365 in

  - Zero the machine (X-axis)

    - Make sure the drill is a bit away from the connector, turn the machine on

    - Move the connector so that the drill takes the smallest amount of material off

    - Zero the screen (same as on Z-axis)

  - Take material off in small X-axis increments by cranking the Y-axis wheel so that the bit moves back and forth

    - J suggests going in 0.02 in increments, but I suggest somewhere around 0.04. You just have to make sure everything is tight so that the connector doesn’t rotate when you mill it flat

  - Take the correct amount of material off

    - Amount will vary depending on measurements of the individual connector

    - Leave 0.3 in of a rim for vertical and one sided horizontal connectors

    - Leave 0.2 in of a rim on each side for a double sided horizontal connector

  - Repeat on the other side

    - Make sure connector and block is going on clean, level surface

    - Make sure connector and block is flush with the clamp

    - Re-zero X-axis, you do not need to re-zero Y-axis

  - Use a metal rasp to remove the burrs

  - The milled connector will look something like this:

OR

### Drilling

  - Use mill machine

  - Change drill bit from milling to drilling

    - Will want something that makes a pilot hole and then a ⅜ in drill bit

  - Mark the correct spot on the flat part of the connector

  - For one ended pieces

    - Put the flat part of connector on top of the metal pieces in the clamp

      - They look like this:

    - Tighten clamp

    - Drill pilot hole, change drill bit, drill actual hole

  - For double ended pieces

    - Put the flat part of connector on smaller square aluminium tubes in the clamp

      - Use as many scrap square tubes as needed to make top clamp-able and flush-ish (try your best)

    - Use same process as one ended pieces

  - Your final product should look something like this:

# Carbon Fibre

### Cutting

  - Wrap in blue tape before cutting, prevents splintering

  - If you cut the piece slightly too long you can sand it down to size

# 3D Printed Pieces

- See Directions for Markforged

- Files are located in folder

- To remove supports, twist then pull